TSD Ringback.  

The following modification are needed to implement tsd ringback feature:

1. SICC Interface ( SIP/CSE) :

Establish call :

I
Active SDP indication – flag.

CallDelivered:

SDP present – flag

Sip/CSE modification.

2. Console new fields in subscriber profile:

Originating Ring Back Enable  

Originating Ring Back Uri

Originating Ring Back Prefix.

3. CSE:

In case Originating Ring Back Enable equal true, add new values of ORBT type to the Invocation table. Values of the new numbers will be Originating Ring Back Uri UserPart  and Originating Ring Back Prefix + Originating Ring Back Uri UserPart .

4. CSE dispatcher:

in case of initial incoming RouteRequest with rUri user part equal to the Invocation number of ORBT type , dispatcher should send SiccAlert message and discard call ( no session creation needed ).

5. CSE SCL :

in TSD state machine the following changes are needed.

Based on the value of ORIG_RING_BACK_OPTION parameter from INIT_PARAM table CSE will issue SiccEstablishCall ( with Active SDP Indication ) to provisioned ORBT Uri. All events from this new ORBT call will be ignored.

ORBT call will be Aborted in the following cases:

TransferCall to B party resulted in SiccDelivered with SDP present flag.


Call to B party established.


Call to B party was not successfully completed ( Busy )


Originating party hangs up.

6. INIT_PARAM table.

New parameter : ORIG_RING_BACK_OPTION 

Values:  NO_RINGBACK, RING_BACK_ON_DIGITS, RING_BACK_ON_NO_SDP, RING_BACK_DURING_DIGIT_COLLECTION.

